The README.docx was written by Zhiming Gao (Phone: 865-946-1339, email: gaoz@ornl.gov) The detailed model are reported in the manuscript of “An Open-Source Code for Modeling and Simulation of Membrane-Based Dehumidification and Energy Recovery.”

Document file

The zipped files include two folders: CODE_MODEL and CASES. In the folder of CODE_MODEL, there are totally fifteen source-code files. The source-code files were written in C++. The functions are explained in the Code function.

In the folder of CASE, there are ten files which include one executable file and 9 input text files for case studies.

- Membrane_Model.exe is an executable file which can be generated from the source codes in the folder of code_model. The used compiler is Microsoft Visual Studio Community 2015. The executable file can be used directly to study any cases with the format of the attached input files.

- 9 input files for case studies include a2a104_s3_cr.txt, a2a520_s3_cr.txt, a2v360_caseA_cr.txt, a2v360_caseB_cr.txt, a2v360_caseC_cr.txt, a2v360_caseD_cr.txt, a2v360_caseE_cr.txt, a2v360_caseF_cr.txt, a2a360_caseG_cr.txt. Here, a2a104_s3_cr.txt and a2a520_s3_cr.txt are the input files for the AIR-AIR cases, and the input file can used to repeat the results reported in Figures 8-10 of the manuscript; and a2v360_caseA_cr.txt et.al. are the input files for the AIR-vapor cases where the permeate side is under the condition of vacuum, and these input file can used to repeat the results reported in Figures 4-7 of the manuscript.

Code function

Cmatric.cpp	: The defined routines are used to allocating memory for any variables used in the open-source codes.

Component.cpp	: The defined Class is used to carry out a general heat exchanger component simulation based segment-by segment methodology for various flow configurations.

HXM.cpp	: The defined Class is used to reanalyze heat exchanger parameters and geometry size, segment size, characteristic length.

Iofile.cpp	: The defined namespace is used to read input data file and generate out data file.

Main.cpp	: This is a Main function to call other functions.

Membrane.cpp	: The namespace of Membrane is defined and is used to calculate membrane mass transport analysis.

MoistAirVapProperty.cpp : The namespace of MoistAirVapProperty is defined and is used to calculate moist air and vapor properties, and their heat transfer coefficients. .

SegmentSolve.cpp	: The defined Class is used to the heat and mass transport process in feed-side and permeate-side flow for each segment. The solving method is Gaussian elimination.

Input parameter introduction

length_dev			: length of device, m

deep_dev			: deep of device, m

height_feedchanel		: height of feedair channel, m

height_permeatechanel	: height of permeateflow channel, m

thick_membrane		: membrane thickness, m

segment_length		: segment number of L-direction

segment_deep			: segment number of D-direction

layernum_membrane		: layer number of membrane

flowtype_device	: flow configuration: 1: parallel uncounterflow; 2:parallel counterflow; 0 or 3 or other: crossflow

poreradius_membrane	: pore radius of membrane,m (need it if membrane_model=0)

porosity_membrane	: porosity of membrane,(-) (need it if membrane_model=0)

tortuosity_membrane	: tortuosity of membrane,(-) (need it if membrane_model=0)

thermocond_membrane	: membrane thermocondivity,kW/m-K(need it if membrane_model=0)

mass_feedflow			: mass flow rate @ feedair, kg/s

temp_feedflow			: feedair temp, K

rh_feedflow			: rh@feedair, (dimensionless)

press_feedflow		: feedair pressure, Pa

mass_permeateflow		: mass flow rate @ permeateflow, kg/s

temp_permeateflow		: peameateflow temp, K

rh_permeateflow		: rh@peameateflow, (dimensionless)

press_permeateflow		: peameateflow pressure, Pa

flowcondition_permeate	: 0:A2V,vapor only in permeateflow; 1:A2A,air in permeateflow

membrane_model		: 0: default model; 1: Dais data; 2 constant value-model

htc_correction		: HTC correction (0-1), (-)

ftc_correction		: FTC correction (0-1), (-)

mt_impact_ht			: 0: not enabled; 1: enabled mass permeated on ht impact

memsurf_correction		: membrane surface deflection (0-inf, 1 means perfectly flat)